# Defining Partial FPGA configurations for near instant PnR

# Research Goals / Problem Statement

## Big picture (more than will be done on this grant, providing context)

Fast mapping for FPGAs

- Decomposition for incremental and parallel/cloud compilation

- Fast assembly of separately compiled (optimized) modules

  - general compilation

    - PRflow

  - datacenter, networking composition

    - vision of assigning various network functions to an FPGA dataplane

    - FYI: we just sent a paper off to NSDI on dataplane mapping -- taking a single P4 program that included calls to Network Functions and decomposing that to run on a heterogeneous array of programmable devices (including CPUs, P4-routers, FPGAs, NPUs)

      - We’ve been programming custom (NF?) boosters in HLS that we compile to FPGAs for this

        - http://ic.ese.upenn.edu/abstracts/faulty_links_netcompute2018.html

## Specific goal of this grant

Open source (symbiflow) sub-fpga (partial reconfiguration) mapping to support vision

- Innovation

  - custom interconnect blocks (possibly replace overlay network)

    - direct pipeline connections, no switching

      - lower overhead interface, more wires, more connectivity

      - Compile to Verilog from application specification showing how compute blocks are connected

    - map in parallel (simultaneous) with leaf computations

      - Each parallel component compile in minutes

    - (maybe) customized switching blocks

      - Could include hybrid direct + switching

  - customized leaf sizes and composition

    - move boundaries to resize capacity

    - customize I/O (add more where needed)

    - hierarchical decompose

      - Split larger section into pieces as needed

  - toward full custom decomposition

- R&D Components

  - describe overlay decomposition

    - support for eventually having tools optimize/select decomposition

  - selecting partition pins (when not relying on vivado)

    - probably requires experimentation to determine rules and strategy

  - extract or constrain PR region for VPR arch

    - Current RoIs and work done is proof of concept

    - Probably need some work to generalize and automate more

    - Extracting to minimal architecture is work to do, but so far nice-to-have

    - May need to be a combination of extraction and constraining

      - Generating an architecture file is slow, so we should avoid having that in the critical path to using a custom overlay

      - Probably some experimentation and optimization here to identify best hybrid strategy

      - May need some cleverness to edit an architecture file for partition pins?

- Mechanics Development (breakdown of tasks/features need to get into SymbiFlow)

  - define and route from partition pins

    - Current work is proof of concept

    - Need to validate and automate further

  - place and route PR region

    - Have working version

  - reserve resources used by static

    - Not touched, but expect to be straight forward

    - Full open-source version of this may not need to deal because we don't let any get borrowed

    - Issue while working with Vivado static regions

  - extract partial bitstream file form VPR P&R results

    - Not done

    - Expect to be straight-forward

    - (but BEL extract in general appears brittle in symbiflow)

  - monolithic bitstream compilation/assembly from separately compiled pieces

    - Avoid need for actual partial reconfiguration or specific requirements of FPGA hardware for partial reconfiguration

    - Could be beneficial for cases where cannot do PR

      - like when vendor locks down in the cloud

    - [this is my understanding of Tim’s offline case]

  - managing bitstream loading/assembly

  - some minimal interfacing to play with static region for PS, DMA, IO on Zynq

    - Probably work like existing RoI scaffold

- Logistics/scoping

  - Target Zynq 7-series

    - Build on current Project X-Ray support

    - Our main model depends on ARM processor for coordination

    - 7020 definitely

      - build on BYU and Ant Micro work?

    - ? larger devices (e.g. 7060) …

      - might require additional work to run fuzzers to get DB

  - ??? guessing Project U-Ray still too far out?

# Description of work, expected outcomes of results

Crossed over into that above.

Expect to be able to:

- Fill in decomposition specification file

- Process the specification file to configure place and route (maybe extract architecture)

- Able to run mapping (synthesis, place, and route) for each PR region independently and get partial bitstreams

  - Target of fast map version in minutes

- Able to load partial bitstreams together to compose larger, functional design